Executed by: Рябов Георгий 424309

Task acc32 for AK-47.

Is_prime

def is_prime(n): """Check if a natural number is prime""" if n < 1: return -1 if n == 1: return 0 for i in range(2, int(n**0.5) + 1): if n % i == 0: return 0 return 1 assert is_prime(5) == 1 assert is_prime(4) == 0 assert is_prime(7) == 1 assert is_prime(8) == 0 assert is_prime(283) == 1 assert is_prime(284) == 0 assert is_prime(293) == 1

Проверка составное ли число иль нет

Конфиг

limit: 100000 state_log_limit: 100000 memory_size: 0x1000 memory_mapped_io: 0x80: [283] 0x84: [] reports: - name: Check results slice: last filter: - state view: ¦ numio[0x80]: {io:0x80:dec} numio[0x84]: {io:0x84:dec} assert: ¦ numio[0x80]: [] >>> [] numio[0x84]: [] >>> [1]

Решение

.data n: .word 0 i: .word 0 const_one: .word 1 .text .org 0x0100 _start: load_addr 0x80 store_addr n load_addr n bltz return_minus_one beqz return_minus_one load_addr n sub const_one beqz return_zero load_imm 2 store_addr i loop_start: load_addr i mul i bvs return_one sub n bgtz return_one load_addr n rem i beqz return_zero load_addr i add const_one store_addr i jmp loop_start return_minus_one: load_imm -1 jmp finish return_zero: load_imm 0 jmp finish return_one: load_imm 1 jmp finish finish: store_addr 0x84 halt

n – проверяемое число

i – счетчик цикла

const_one – константа (инкремент/проверки)